Egemen Çelik

2220357034

25/04/2026

ELE432 Advanced Digital Design

Homework 2

1) I spent 5 hours on this homework.

2) “aludec” and “instrdec” were given in the homework 2 pdf.

RTL Schematic is given below.

Figure 1 RTL Schematic

File: controller.sv

| /* Egemen Celik 2220357034 ELE432 HomeWork 2 : Multicycle_Processor_Controller controller.sv code modified from Harris&Harris book */ module controller ( input logic clk, input logic reset, input logic [6:0] op, input logic [2:0] funct3, input logic funct7b5, input logic Zero, output logic [1:0] ImmSrc, output logic [1:0] ALUSrcA, ALUSrcB, output logic [1:0] ResultSrc, output logic AdrSrc, output logic [2:0] ALUControl, output logic IRWrite, PCWrite, output logic RegWrite, MemWrite ); // internal wires logic Branch; logic PCUpdate; logic [1:0] ALUOp; assign PCWrite = (Zero & Branch) ¦ PCUpdate; | // decoder logic maindec md ( clk, reset, op, Branch, PCUpdate, ALUSrcA, ALUSrcB, ResultSrc, AdrSrc, IRWrite, RegWrite, MemWrite, ALUOp ); aludec ad ( op[5], funct3, funct7b5, ALUOp, ALUControl ); instrdec id ( op, ImmSrc ); endmodule |
| --- | --- |

File: maindec.sv

| /* Egemen Celik 2220357034 ELE432 HomeWork 2 : Multicycle_Processor_Controller maindec.sv */ module maindec ( input logic clk, input logic reset, input logic [6:0] op, output logic Branch, output logic PCUpdate, output logic [1:0] ALUSrcA, output logic [1:0] ALUSrcB, output logic [1:0] ResultSrc, output logic AdrSrc, output logic IRWrite, output logic RegWrite, output logic MemWrite, output logic [1:0] ALUOp ); // State encoding typedef enum logic [3:0] { S0_FETCH = 4'b0000, S1_DECODE = 4'b0001, S2_MEMADR = 4'b0010, S3_MEMREAD = 4'b0011, S4_MEMWB = 4'b0100, S5_MEMWRITE = 4'b0101, S6_EXECUTER = 4'b0110, S7_ALUWB = 4'b0111, S8_EXECUTEI = 4'b1000, S9_JAL = 4'b1001, S10_BEQ = 4'b1010 } state_t; // State variables state_t current_state, next_state; // Opcode definitions localparam logic [6:0] OP_LW = 7'b0000011; localparam logic [6:0] OP_SW = 7'b0100011; localparam logic [6:0] OP_RTYPE = 7'b0110011; localparam logic [6:0] OP_ITYPE = 7'b0010011; localparam logic [6:0] OP_JAL = 7'b1101111; localparam logic [6:0] OP_BEQ = 7'b1100011; // Sequential State transition logic always_ff @(posedge clk or posedge reset) begin : Sequential_State if (reset) current_state <= S0_FETCH; else current_state <= next_state; end // Combinational State transition logic always_comb begin : Combinational_State // Default values next_state = current_state; case (current_state) S0_FETCH: next_state = S1_DECODE; S1_DECODE: case (op) OP_LW, OP_SW: next_state = S2_MEMADR; OP_RTYPE: next_state = S6_EXECUTER; OP_ITYPE: next_state = S8_EXECUTEI; OP_JAL: next_state = S9_JAL; OP_BEQ: next_state = S10_BEQ; default: next_state = S0_FETCH; endcase S2_MEMADR: case (op) OP_LW: next_state = S3_MEMREAD; OP_SW: next_state = S5_MEMWRITE; default: next_state = S0_FETCH; endcase S3_MEMREAD: next_state = S4_MEMWB; S4_MEMWB, S5_MEMWRITE, S7_ALUWB, S10_BEQ: next_state = S0_FETCH; S6_EXECUTER, S8_EXECUTEI, S9_JAL: next_state = S7_ALUWB; default: next_state = S0_FETCH; endcase end | // Output logic always_comb begin : Outputs // Default values Branch = 1'b0; PCUpdate = 1'b0; ALUSrcA = 2'b00; ALUSrcB = 2'b00; ResultSrc = 2'b00; AdrSrc = 1'b0; IRWrite = 1'b0; RegWrite = 1'b0; MemWrite = 1'b0; ALUOp = 2'b00; case (current_state) S0_FETCH: begin AdrSrc = 1'b0; IRWrite = 1'b1; ALUSrcA = 2'b00; ALUSrcB = 2'b10; ALUOp = 2'b00; ResultSrc = 2'b10; PCUpdate = 1'b1; end S1_DECODE: begin ALUSrcA = 2'b01; ALUSrcB = 2'b01; ALUOp = 2'b00; end S2_MEMADR: begin ALUSrcA = 2'b10; ALUSrcB = 2'b01; ALUOp = 2'b00; end S3_MEMREAD: begin ResultSrc = 2'b00; AdrSrc = 1'b1; end S4_MEMWB: begin ResultSrc = 2'b01; RegWrite = 1'b1; end S5_MEMWRITE: begin ResultSrc = 2'b00; AdrSrc = 1'b1; MemWrite = 1'b1; end S6_EXECUTER: begin ALUSrcA = 2'b10; ALUSrcB = 2'b00; ALUOp = 2'b10; end S7_ALUWB: begin ResultSrc = 2'b00; RegWrite = 1'b1; end S8_EXECUTEI: begin ALUSrcA = 2'b10; ALUSrcB = 2'b01; ALUOp = 2'b10; end S9_JAL: begin ALUSrcA = 2'b01; ALUSrcB = 2'b10; ALUOp = 2'b00; ResultSrc = 2'b00; PCUpdate = 1'b1; end S10_BEQ: begin ALUSrcA = 2'b10; ALUSrcB = 2'b00; ALUOp = 2'b01; ResultSrc = 2'b00; Branch = 1'b1; end endcase end endmodule |
| --- | --- |

3) The controller.tv file was using a RISC-V standard different that the one in the Harris & Harris book and the homework file. I updated the expected ALUControl bits to match the Harris & Harris’s description. The updated file can be found in the GitHub repository.

Example:

| //op[6:0]_funct3[2:0]_funct7b5_Zero_ImmSrc[1:0]_ALUSrcA[1:0]_ALUSrcB[1:0]_ResultSrc[1:0]_AdrSrc_ALUControl[2:0]_IRWrite_PCWrite_RegWrite_MemWrite // add |  |
| --- | --- |
| old | updated |
| 0110011_000_0_0__XX_00_10_10_0_010_1_1_0_0 // Fetch 0110011_000_0_0__XX_01_01_00_0_010_0_0_0_0 // Decode 0110011_000_0_0__XX_10_00_00_0_010_0_0_0_0 // ExecuteR 0110011_000_0_0__XX_00_00_00_0_010_0_0_1_0 // ALUWB | 0110011_000_0_0__XX_00_10_10_0_000_1_1_0_0 // Fetch 0110011_000_0_0__XX_01_01_00_0_000_0_0_0_0 // Decode 0110011_000_0_0__XX_10_00_00_0_000_0_0_0_0 // ExecuteR 0110011_000_0_0__XX_00_00_00_0_000_0_0_1_0 // ALUWB |

Testbench Waveform Output:

Questa Transcript Output:

As you can see after the issue with the ALUControl bits is resolved there are no errors and everything works as expected.

4) The whole Quartus project directory along with all of the files of interest are uploaded inside separate folders along with this file in both Word and PDF forms.

GitHub link:	https://github.com/20egemen04/Multicycle_Processor_Controller